Week 3 Hands on Checklist:

| Image Optimization |  |
| --- | --- |
| Obtain a PLAX view and utilize ZOOM function on the aortic valve |  |
| Obtain a PSAX view and utilize ZOOM on the pulmonic valve |  |
| Calculation of Cardiac Output from LV measurements |  |
| Obtain a PLAX view, FREEZE and MEASURE the diameter of the aortic outflow tract using the inner edge to inner edge. Measurement is during maximum opening of the valve during mid-systole. |  |
| Obtain a A5C view, place pulse wave doppler proximal to the aortic valve and obtain a tracing of the envelope |  |
| Calculate stroke volume and cardiac output based on above values |  |
| Repeat in a high output state |  |
| Calculation of Cardiac Output from RV measurements |  |
| Obtain a PSAx view |  |
| Measure diameter of distal RV outflow tract using inner edge to inner edge at end diastole, just proximal to the valve |  |
| Place pulse wave doppler through the RV outflow tract, proximal to the pulmonary valve and obtain a RV VTi tracing |  |
| AoVTI Respiratory Variation |  |
| Obtain a A5C view and AoVTI signal |  |
| Increase sweep speed, freeze |  |
| Compare maximum and minimum aortic outflow velocities |  |
| Compare maximum and minimum aortic outflow velocities in a high output state |  |